# Map Standards and OCAD

Gary Carroll. Work in progress – September 2002

## Introduction

OCAD is a Windows CAD (Computer Aided Drafting) package tailored for making orienteering maps. OCAD is capable of producing very detailed high quality orienteering maps. Rogaining maps are relatively straightforward. OCAD has been used to produce most WARA rogaine maps since 1996.

WARA are currently using OCAD 7 which will run on any 32 bit Windows Operating System like Windowns 95/98 and NT Windows 2000.

Proper use of Computer Aided Drafting (CAD) will greatly enhance the quality of WARA rogaine maps:

- It is much easier to produce a computer generated map than a traditional tech-drawing style map unless you have drafting experience. Mistakes become obvious on screen or on draft print outs. The greatest problem with pen and film methods of producing maps is keeping the drafting sheets layers aligned to avoid registration problems during printing. With CAD this is not an issue.

- Draft copies of the actual map that the competitors will be using can be easily produced. This will greatly aid the vetting process, as the vetters will use the map the competitors will use. The importance of having the vetters use the same map as the competitors can not be understated.

Experienced computer draftsmen may prefer to use MicroStation. For advice on using MicroStation please call Vince Harding (9363 6403). Although this guide does not directly address MicroStation, many of the more general comments made here are still relevant.

This guide is not meant to replace the substantial OCAD manual. Instead, it is designed to aid you - the apprentice WARA cartographer - in producing a WARA style rogaine map. This guide can not answer every question possible. Every map will have it's own peculiarities. We are very much at an early stage of our experiences with OCAD. So, if you can suggest any improvements to this guide or to our way of using OCAD please do so.

Jim Langford, Phil Dufty and Gary Carroll are the nominated "experts" in using OCAD. Please refer your questions to them. Jim can be contacted on 9387 6347, Phil on 9459 1173 and Gary on 9307 6830.

## Important: Licence Conditions

WARA has a single user licence for OCAD. It is not a site licence for the whole rogaining association. You must delete OCAD when you are finished using it.

## Approach to Making Your Map

Please read the OCAD manual, and be prepared to spend some time learning, and making mistakes.

It would be wise to save multiple versions of your map as your work progresses. If something goes wrong you can revert to a previous version without the need to start again from scratch.

Save copies of the map onto diskettes and hide them, in case your computer dies or is stolen!

## Printing Process

There are two ways of providing our commercial printer with your finished product.

• You can send a copy of the OCAD file, and any unusual fonts that you have used, to WARA's preferred printer Bunbury Print, 9 Zaknic Place  Bunbury 6230, Phone (08) 9791-5116 or Fax  (08) 9791-5117. You can e-mail your OCAD data file to byprint@gateway.net.au. Bunbury Print are now very experienced in printing OCAD rogaining maps. Their maps are good quality and economical. We are currently recommending that you get 600 maps printed for Autumn and Spring 12 hour events and both 24 hour events, and 300 maps for the 6 hour and upside down events. However please check with Gary Carroll before specifying the number of maps to be printed. A week before you have you map printed, you should send Bunbury Print a draft copy of your map especially if it contains any unsusual fonts or graphics.

• You can use the WARA ink jet printer to print colour separations onto drafting sheets, which are sent to the commercial printer. This method is NOT recommended for OCAD users, however MicroStation users must do this as our preferred printer does not have MicroStation.

You should warn Bunbury Print 3 weeks before the rogaine that they are about to receive some work. The finished map should be submitted around the same date as the close of entries, which is normally 10 days before the rogaine.

## Composite and Separation Printing

• Composite: This is the way you will print any draft maps you produce yourself, such as to an inkjet printer. All the colours will be printed at the same time. The order of the colours in OCAD’s colour table will be followed. Red features are on top of all others, then black features, then blue features and brown features are at the bottom.

• Separations. The commercial printer will use colour separations. All the colours will be printed over the top of each other. It is therefore important to have details (control numbers, road names) away from areas of dense contour lines.

OCAD has the ability to mask out colours. Do not use this feature!

## Standards

The following standards should apply to all rogaine maps. Standards are important so competitors use maps that are consistent from event to event. You can deviate from these standards, but please seek approval from the committee.

If you are using OCAD, you do not need to worry about these standards below. The base map that you will be supplied with already conforms to the standards.

- (This section is incomplete)

Brown features

Ask the printer to use a dark brown.

Contours: 0.12 mm wide lines

Blue Features

Dams: should be a solid blue fill. Small dams should be a square with 1mm sides.

Major Watercourses: 0.25 mm wide line.

Minor Watercourses: 0.20 mm wide line.

Watercourse direction arrows: 0.20 mm wide lines.

Waterdrop circle: 0.25 mm wide lines, diameter 4mm.

Waterdrop Upper case “W”in 14 pt Arial.

North Lines: 0.25 mm wide. 30 or 40mm (1.5km or 2km on 1:50,000 map) separation.

Black Features

Sealed Roads: 0.45mm wide, solid lines.

Major unsealed Roads: 0.35mm wide, dashed line with 0.70mm gaps.

Minor Tracks: 0.25 mm wide, dashed line with 0.50mm gaps.

Small buildings:  should be a solid black fill. Square with 1mm sides.

Red Features

Control Circles: 0.35 mm wide lines, diameter 6mm

Control numbers: 18 pt Arial

Hash House: Equilateral triangle. 0.35 mm wide lines. 7mm sides.

OOBs - Small areas: 0.25mm wide lines. 2mm separation. 45 degree angle. Border 0.30 mm wide.

OOBs – Large areas: 0.25 mm wide lines. 6mm separations. 45 degree angle. Border 0.30 mm wide.

Other colours are sometimes used such as green to mark pine forests.

See section 8.4 for a list of the attributes of a WARA rogaine map.

## Tips of the Trade!

Some tips of the trade which may not be obvious:

Optimise your data file

Use the Extra menu’s repair/optimise option frequently. It will clean up and shrink the size of your map file enormously.

Don’t change the symbol characteristics or colours

Do not alter the line thickness or colours of symbols to suit your home PC’s printer, and then submit the altered version to Bunbury Print.  Stick with the standard thickness and the map will be printed correctly at the commercial printer.

Finding the declination

The declination is the different between magnetic north and grid north. The data you receive will be in grid north, so the magnetic north lines on the map will have to be tilted slightly. In the areas that WARA uses this is normally between 1.5 and 4 degrees west of north. The declination is sometimes printed on the map.

You can find the declination for a particular site and date on the Australian Geoscience web page:

http://www.agso.gov.au/oracle/geomag/agrfform.html

This site requires the latitude and longitude of your site, and this can be obtained from the Geoscience Australia website http://www.ga.gov.au/map/names. This site includes the latitude of an enormous number of locations including minor creeks and tracks.

Making North Lines

This is very easy in OCAD-7.

- Select the North Line Symbol

- Select the Extras menu and the Gridlines option.

- Enter 40mm for the horizontal spacing (for a 2km spacing)

- Enter 0 for the horizontal offset.

- Enter the declination in angle. This will normally be a positive number, about 2.5 is typical.

- Make sure the Create Vertical Lines is true and that Create Horizontal lines is false.

- Click Okay.

## Your Responsibilities

After the rogaine is complete, please return all maps, drafting film, aerial photographs purchased by WARA to Vince Harding.

Please provide a copy of the data (MicroStation or OCAD) of your finished map to Gary Carroll. You might also produce any other useful info such as letters you have written, clue sheets, details on landholders such as their contact phone numbers and their attitude towards rogaining. This info will be archived on CDs for future use.

## The Gory Details

The methods described from here are not intended for use by setters. Setters should contact Gary Carroll who will deliver the data for an event already in OCAD format. This section is purely for purposes of having our methods documented.

### Purchasing Data from DOLA

Data for maps should be purchased from DOLA at Midland. The contacts there are Marty Stamatis or David Chiang (Marty_Stamatis@ notes.dola.wa.gov.au and David_Chiang@notes.dola.wa.gov.au ) (9273 7373). When you purchase the data please inform them that you want the data to be in DXF format. DXF files are a standard format for CAD data and can be read by OCAD and MicroStation. DOLA can provide a great deal of information for an area. Most of it is irrelevant for a rogaining map. The irrelevant data can be digitally thrown away at a later stage.

There are many ways of obtaining the data. You can use diskettes, a writable CD, or using ftp (file transfer protocol) via the Internet.

If you contact someone at DOLA and they are not familiar with how to produce OCAD readable DXF files, then use these instructions which came from Marty Stamatis of DOLA:

Our method of supply is as follows,

1) Export from Microstation DGN format to DXF using Microstation

2) The DXF version supplied is 12 (current export level within Microstation)

3) DGN files are converted from 3D to 2D

4) Complex line strings in the DGN file are dropped to line strings

Points 3 and 4 are performed because many of out users do not use Autocad to process the DXF data, hence, some systems e.g. ocad can not handle complex lines in the DXF format.

### Ordering Data

There are a two ways of defining the area of data to purchase. How we order the data depends on the location, how well your area fits the standard 1:25,000 sheets and what other data we have for nearby areas.

- Be specific and provide the exact map numbers required. These will be the 1:25,000 sheet numbers. Such as Quindanning , 2231-IV-NE.

- Ask for the data in terms of Australian Map Grid (AMG) coordinates. Eg: East Side: 490000, North Side: 6345000, West Side: 470000, South Side  6325000.

- DOLA charge for their labour. The simpler it is for them, the cheaper it is for us.

- Typically data for a map will cost $300 to $400.

- We can also obtain data which is outside of the normal 1:25,000 map area. To produce data in this format the staff at DOLA will need to convert a DEM (Digital Elevation Model). This will add up to $200 to the cost of the data. Depending on busy the DOLA staff are, this may take a few weeks to do.

### Import the data into OCAD

- To load the data into OCAD you will need to set up a "CRT" file. This is a text file that will tell OCAD which DXF "layer" should be converted to which OCAD symbol, what scale to use and it defines the center of the map.

- Typically, a CRT file looks like:

- SCALE 50000

- ZEROX 452500

- ZEROY 6420000

- 505.0 3

- 516.0 4

- 550.0 5

- 551.0 6

- 527.1 7

- and so on…

- The centre of the map you are importing is defined by the ZEROX and ZEROY lines of the .crt file. These are AMG grid coordinates that define the centre of the map for the data you are importing. These can be found by looking at the DXF file your are importing or from your paper copy of the map. If the ZEROX and ZEROY are too far off centre OCAD will reject data (contours, roads, etc) that are close to or are off the edge of OCADs "page".

- The DXF file is simply a long text file. Look for lines near the top of the DXF file like:

- $EXTMAX

- 10

- 465143.74

- 20

- 6362571.54

- In this example ZEROX is 465144 and ZEROY 6362572.

- To import the data into OCAD, perform the following steps.

- • Make a copy of the standard OCAD map WARA.OCD and name it for your event. Keep the OCD extension.

- • Open your event map, and import the data using the Extras menu's Import DXF option. You will be asked whether you wish to use a CRT file. Answer "yes" to this and choose the CRT file you have written.

- You are almost there. If you can not see any map detail, then choose to view the entire map.

There are now a few fiddly problems to deal with such as the following.

- Some features will appear to be grey. These features are not defined in the crt file. Some will be obvious and some will not. As we (I) get more experience (time) to play with OCAD the number of grey features will be reduced. They exist in the DXF file but there is no translation for them in the CRT file. Have a look at the grey features. These may include features such as spot heights and springs. Some of the data may be useful - for example silos could be mapped as buildings. Some will be windmills or springs. Some will be map artifacts like grid coordinates or north symbols that can be removed.

- You may find that dams are defined in a variety of styles. This is a huge problem on farmland maps. The style of maps used depends on the age of the data.

- You may find that the features at the edges of the original 1:25000 sheets do not join up perfectly. This has to be fixed manually. This is extremely boring and tedious.

- Some large areas features (pine forests, bare rocks, lakes) may have been broken up into multiple segments. This will result in OCAD being unable to properly colour in the "inside" of the feature properly. Use OCAD’s merge tool to join the broken feature into one object. This is also extremely boring and tedious.

- Sometimes buildings get drawn as a collection of lines rather than the normal square symbol. As the building is made up of line segments and not point features you can not automatically translate and correct these. Again, it is a manual task.

All the common "grey" features will be given the same negative symbol number. For example all spot heights could be -1. The cross hairs for the spot height could be symbol –2, all grid references –3 and so on. If you choose to delete features with symbol -1 then you will delete only the spot heights. Use the Extras menu’s Delete by symbol option.